# Intro to Audio Mixing

Logic Session we will work though today is available here for download:

https://drive.google.com/file/d/1voTiR91ezcdVg04ZDi5TENgO9HwLmuxr/view?usp=sharing

Click on the three-dots and then “download”

## What is Mixing?

“We are mixing engineers, but more importantly: we are sonic artists.” — Roey Izhaki, Mixing Audio

Mixing is the process of combining individual recorded tracks into a finished single piece of audio. It is both a technical craft and a creative act.

## Owsinski’s Six Elements of a Mix

Bobby Owsinski’s framework from The Mixing Engineer’s Handbook organizes everything in a mix into six elements:

## 1. Organize

Name, colour-code, and group your tracks. Route related channels to subgroup busses (e.g., all drums to a “Drums” bus). create a mix bus and global reverb bus. Set yourself up for clarity or signal flow and organization before you touch any processing.

Signal Flow

Key concept: Use inserts for processing that’s unique to a channel (EQ, compression). Use sends for effects shared across multiple channels and additive effects (reverb, delay). This creates a cohesive sense of space, finer control of the sound, and saves CPU.

## 2. Gain Stage

Gain staging means setting healthy and balanced signal levels across all tracks. The goal is to keep levels in the “sweet spot”: loud enough for a clean signal, quiet enough to avoid clipping and leave headroom.

Gain staging can be achieved with a gain plugin at the top of the processing chain or other means depending on the DAW. Leave your faders at unity, those will be used later on!

### Best Practices

- Start at the source: Set your channel input gain so peaks sit around -18 to -12 dBFS — this leaves plenty of headroom and keeps plugins operating in their sweet spot

- Unity gain through plugins: After adding EQ or compression (stage 4 and 5), check that the output level roughly matches the input level. Louder isn’t better — it just tricks your ears and makes the comparison harder.

- Watch the mix bus: Keep your master fader at 0 dB. If the mix bus is clipping, turn individual channels down rather than pulling the master fader down.

- Leave Headroom: On your mix bus aim for peaks below -6 dBFS — this leaves room for mastering.

## 3. Static Mix & Panning

Use the faders and pan only — no plugins. Get the balance right first. This is the foundation of your mix.

Some typical panning: - Centre: Lead, bass, and primary rhythm. - Sides: Harmony, secondary elements, texture, percussion. consider hard panning and using the entire width available! - Check in mono: If panned elements disappear in mono, there may be phase issues.

## 4. EQ (Equalization)

EQ shapes the frequency content of a sound — what you hear as tone, brightness, warmth, or muddiness.

### The Frequency Spectrum

### EQ Controls

| Parameter | What It Does | Think of It As… |
| --- | --- | --- |
| Frequency | Selects the centre frequency to boost or cut | Where on the spectrum you’re working |
| Gain | How much you boost (+) or cut (−), in dB | How much you’re turning that frequency range up or down |
| Q (Bandwidth) | How wide or narrow the affected range is | Low Q = broad and gentle; High Q = narrow and surgical |

### Common Filter Types

| Filter | Shape | Typical Use |
| --- | --- | --- |
| Bell (Peak) | Symmetrical boost/cut around a centre frequency | General-purpose tonal shaping — the most common EQ filter |
| High-Pass (HPF / Low Cut) | Passes highs, removes everything below a cutoff | Cleaning low-end rumble from non-bass instruments |
| Low-Pass (LPF / High Cut) | Passes lows, removes everything above a cutoff | Taming harsh high-frequency content or creating a “distant” effect |
| High Shelf | Boosts or cuts all frequencies above a set point | Adding “air” or reducing brightness across the top end |
| Low Shelf | Boosts or cuts all frequencies below a set point | Adding warmth or reducing low-end weight |
| Notch (Band-Reject) | Very narrow cut at a specific frequency | Removing a specific resonance or hum (e.g., 60 Hz electrical hum) |

A parametric EQ gives you independent control of frequency, gain, and Q for each band — it is the standard EQ type in most DAWs and the most flexible tool for mixing.

### Owsinski’s Six Trouble Frequency Areas

| Frequency | Problem | Solution |
| --- | --- | --- |
| ~200 Hz | Mud, boominess | HPF or gentle cut |
| 300-500 Hz | Boxiness | Narrow cut |
| ~800 Hz | Cheap, hollow | Narrow cut |
| 1-1.5 kHz | Nasal, honky | Narrow cut |
| 4-6 kHz | Harsh, sibilant | Gentle cut or de-esser |
| 10 kHz+ | Hiss, sibilance | Shelf cut or de-esser |

### Best Practices of EQ

- Subtractive first: Cut problem frequencies before boosting desired ones — introduces fewer artifacts (Senior, Mixing Secrets)

- HPF on nearly everything: Unless it’s bass or kick, high-pass filter to remove unnecessary low-end rumble.

- Cut narrow, boost wide: Surgical cuts target problems and broad boosts sound more natural

- Don’t use it if it doesn’t need it!

## 5. Compression

Compression controls dynamic range — the difference between the loudest and quietest parts of a signal.

### Key Parameters

| Parameter | What It Does | Think of It As… |
| --- | --- | --- |
| Threshold | Level where compression begins | The “trigger point” |
| Ratio | How much to reduce above threshold | How aggressively the dynamic range is compressed |
| Attack | How quickly compression engages | Fast = catches transients; Slow = lets transients through |
| Release | How quickly compression stops | Fast = can produce a pumping effect; Slow = smooth |
| Makeup Gain | Boosts output to compensate for reduction | Restoring perceived volume |

You can think of a compressor kind of like an automated fader that brings down the level past a certain loudness.

### Best Practices

- Start gentle: Low ratio (2:1-4:1), moderate threshold, and adjust from there

- Listen, don’t just look at meters: Use your ears, not the gain reduction display

- Compression is a balance tool: “From a mix perspective, the primary purpose of compression is to achieve a stable balance.” — Mike Senior, Mixing Secrets

- The fader instability diagnostic: If you keep adjusting a fader, that track probably needs compression (Senior)

## 6. Reverb / FX

Reverb can create a global sense of space but can also be used as an effect on a single track or subgroup. Today we will focus on the idea of a global reverb that helps to create a sense of shared space and coherence to a mix.

### Key Parameters

| Parameter | What It Does |  | Tip |
| --- | --- | --- | --- |
| Pre-delay | Gap before reverb begins | Longer = keeps lead element clear and upfront |  |
| Decay time | Length of reverb tail | Consider matching to song tempo — reverb can be rhythmic |  |
| Wet/dry | Blend of effect vs. original | Less is usually more |  |
| Type | Hall, room, plate, spring, etc. | Each has a character; experiment and get to know the sound of these different types |  |

### Best Practices

- Use sends, not inserts for additive effects like reverb: Route multiple channels to a shared reverb bus with 100% wet and 0% dry signal to control how much reverb is added to the mix and cohere various parts together into the same virtual space

## 7. Refine

Revisit your balance. Make small tweaks to fader levels, EQ, and compression now that all the processing is in place. This might also be a nice place to have a little break and step away from the mix before returning with fresh ears.

## 8. Check

- Reference tracks: Compare your mix against professional mixes in a similar genre.

- Mono check: Collapse to mono to check for phase issues; can you still hear everything?

- Low volume: Does the balance hold at quieter levels?

- Different systems: Headphones, laptop speakers, car — your mix should translate.

## 9. Automate

Volume rides, effect sends, panning movement, filter sweeps — automation is the final stage and a great place to get creative.

## Free Resources

| Resource | What It Is | URL |
| --- | --- | --- |
| cambridge-mt.com | 500+ free multitrack sessions for mixing practice | cambridge-mt.com/ms/mtk/ |
| iZotope Learn Hub | Articles and guides on mixing fundamentals | izotope.com/en/learn |
| Produce Like A Pro (YouTube) | Full mixing tutorials with free multitracks | YouTube |

## Recommended Reading

- Bobby Owsinski — The Mixing Engineer’s Handbook (5th ed., 2022) — Accessible framework + engineer interviews

- Mike Senior — Mixing Secrets for the Small Studio (2nd ed., 2019) — Practical, budget-friendly, hands-on

- Roey Izhaki — Mixing Audio (4th ed., 2024) — Comprehensive technical reference

- William Moylan — The Art of Recording (2002/2015) — Deep analytical and aesthetic perspective

- David Miles Huber — Modern Recording Techniques (9th ed., 2017) — Broad survey of the full recording chain